ACL Continuum Modeling Write Up

Ellen Klonowski

2/21/2022

This study is intended to provide a strategy to represent the anterior cruciate ligament as a continuum that has a structural response consistent with literature. Various commonly used material properties will be demonstrated to represent the structural response of the ACL. The role of material coefficients will be examined along with a method for in silico testing the ACL to further understand its structural response to change in material properties. The method will be applied to several ACLs from the Open Knee(s) cohort.

Based on the current progress of the paper, a force-displacement model to represent the ACL for this study will based on a study by Woo et al. The study measures tensile properties of ACL with the femur and tibia still intact during experimentation. Effect on age and orientation were considered in the study therefore we can correlate them with the age group of the Open Knees specimens. The average stiffness value from the anatomical orientation from the age group of the specimen we are working with will be used as our baseline for calibrating stiffness of the ACL. These values can be found in section 1g of the initial documentation by Ariel Schwartz.

These stiffness values from the force-displacement curve mentioned above will be assigned to their appropriate specimen age group to perform in silico experimentation and evaluate its compression-tension response. Material parameters to represent the uncoupled  transversely isotropic Mooney Rivlin model are established from the work completed by Peña et al. Another item necessary for this modeling the ACL is to calibrate the stiffness of the ligament using the K:c1 recommended in literature (Weiss et al). This should demonstrate that it is impossible to eliminate compressive reaction in a ligament.

A mesh convergence study was completed using oks specimens 001-009. See document referenced for detailed results. Geometry for the models exists in a repository and a sampling rate of 6 is acceptable for all specimens (noted: except for SR4 oks008). This convergence study is completed to confirm a recommended density for future models, and an extensive analysis won’t be necessary each time. Some results exist in the early stages of this study. Influence of material coefficients using oks001 results are located here.

Various combinations of assigned material property should be investigated as listed in section 4 of the Action Items list of the documentation. As material properties are adjusted and explored the “best” model will be chosen and repeated for remaining oks specimens. Repeatable results across many ACL geometries indicates that the optimal material properties are assigned. If there is no best result to draw a conclusion, we can recommend a method for determining the ratio that should be used for checking various structural responses. Discussion points for the paper from early investigation are located at the end of the referenced document.